SupplyOn AirSupply Supplier EDI Information

TECHNICAL GUIDE for a M2M connection to SupplyOn

BoostAeroXML V2.10 message format

VMI Self Billing Invoice

AirSupply GUI Scheduler Message Type:

50 SBI / Self Billing

Document Owner: SupplyOn PM SCM Aero

Version: 1.2

Date: 17.01.2013

Document available in English (only)

History

| Version | Date | Author | Change Log |
| --- | --- | --- | --- |
| 1.0 | 16 Mai 2011 | Danny Badur Georg Büxel | First version |
| 1.1 | 04 Juli 2011 | Georg Büxel | 2 - Changed field length of asram:SpecifiedTransactionIdentifier |
| 1.2 | 17.01.2013 | PM SupplyOn AG | Section 1 Preface added |

Index

1	Preface	5

2	Purpose of the document	5

2.1	Self Billing Invoice Definition	5

2.2	Preliminary Remarks	5

2.3	Composition of the specification	6

2.4	Composition of the XML message graphics	6

2.5	BoostAero Attribut Codes Lists	7

2.6	Glossary	7

3	as:CIExchangedDocumentContext	10

3.1	asram:ApplicationSpecifiedCIDocumentContextParameter	10

3.1.1	asram:SpecifiedCIDocumentVersion	11

4	as:CIBISupplyChainTradeTransaction	12

4.1	asram:ApplicableCIBIHSupplyChainTradeAgreement	13

4.1.1	asram:BuyerCITradeParty	13

4.1.1.1	asram:SpecifiedCITaxRegistration	14

4.1.1.1.1	VATIdentNo	14

4.1.1.1.2	TaxNumber	14

4.1.1.2	asram:PostalCITradeAddress	16

4.1.1.2.1	asram:Geo-CoordinateIdentificationCIGeographicalCoordinate	18

4.1.1.3	asram:DefinedCITradeContact	19

4.1.1.3.1	asram:EmailURICIUniversalCommunication	20

4.1.1.3.2	asram:FaxCIUniversalCommunication	20

4.1.1.3.3	asram:TelephoneCIUniversalCommunication	20

4.1.2	asram:SellerCITradeParty	21

4.1.2.1	asram:SpecifiedCITaxRegistration	21

4.1.2.1.1	VATIdentNo	21

4.1.2.1.2	TaxNumber	22

4.1.2.2	asram:PostalCITradeAddress	23

4.1.2.2.1	asram:Geo-CoordinateIdentificationCIGeographicalCoordinate	25

4.1.2.3	asram:DefinedCITradeContact	26

4.1.2.3.1	asram:EmailURICIUniversalCommunication	27

4.1.2.3.2	asram:FaxCIUniversalCommunication	27

4.1.2.3.3	asram:TelephoneCIUniversalCommunication	27

4.2	asram:ApplicableCIBIHSupplyChainTradeSettlement	28

4.2.1	asram:AcceptanceReferencedCIReferencedDocument	29

4.2.2	asram:InvoiceeCITradeParty	30

4.2.2.1	asram:SpecifiedCITaxRegistration	31

4.2.2.1.1	VATIdentNo	31

4.2.2.1.2	TaxNumber	31

4.2.2.2	asram:PostalCITradeAddress	32

4.2.2.2.1	asram:Geo-CoordinateIdentificationCIGeographicalCoordinate	34

4.2.2.3	asram:DefinedCITradeContact	35

4.2.2.3.1	asram:EmailURICIUniversalCommunication	36

4.2.2.3.2	asram:FaxCIUniversalCommunication	36

4.2.2.3.3	asram:TelephoneCIUniversalCommunication	36

4.2.3	asram:PayeeCITradeParty	37

4.2.3.1	asram:SpecifiedCITaxRegistration	38

4.2.3.1.1	VATIdentNo	38

4.2.3.1.2	TaxNumber	38

4.2.3.2	asram:PostalCITradeAddress	39

4.2.3.2.1	asram:Geo-CoordinateIdentificationCIGeographicalCoordinate	41

4.2.3.3	asram:DefinedCITradeContact	42

4.2.3.3.1	asram:EmailURICIUniversalCommunication	43

4.2.3.3.2	asram:FaxCIUniversalCommunication	43

4.2.3.3.3	asram:TelephoneCIUniversalCommunication	43

4.2.4	asram:SpecifiedCITradePaymentTerms	44

4.2.5	asram:SubtotalCalculatedCITradeTax	44

4.3	asram:IncludedCIBILSupplyChainTradeLineItem	46

4.3.1	asram:SpecifiedCITradeProduct	47

4.3.1.1	asram:InformationCINote	48

4.3.1.2	asram:IndividualCITradeProductInstance	49

4.3.2	asram:SpecifiedCIBILSupplyChainTradeAgreement	50

4.3.2.1	asram:ProgramCIMissionProject	50

4.3.2.2	asram:AdditionalReferencedCIReferencedDocument	51

4.3.2.3	asram:BuyerCITradeParty	51

4.3.2.3.1	asram:SpecifiedCILegalOrganization	52

4.3.2.3.2	asram:DefinedCITradeContact	52

4.3.2.4	asram:SellerCITradeParty	53

4.3.2.4.1	asram:DefinedCITradeContact	54

4.3.2.5	asram:AgreedPriceProductCITradePrice	54

4.3.3	asram:SpecifiedCIBILSupplyChainTradeDelivery	56

4.3.3.1	asram:InformationCINote	57

4.3.3.2	asram:DespatchAdviceReferencedCIReferencedDocument	57

4.3.3.3	asram:ConsumptionCISupplyChainSchedule	58

4.3.3.3.1	asram:ActualCISupplyChainEvent	58

4.3.3.4	asram:ShipToCITradeParty	59

4.3.4	asram:SpecifiedCIBILSupplyChainTradeSettlement	60

4.3.4.1	asram:SpecifiedCITradeSettlementMonetarySummation	60

4.3.4.2	asram:SpecifiedCITradeAllowanceCharge	61

4.3.4.3	asram:ApplicableCITradeTax	61

5	as:CIIExchangedDocument	63

5.1	asram:RecipientCITradeParty	64

5.2	asram:SenderCITradeParty	65

# Preface

It is strongly recommended to read the document “Read Me first – BoostAero General information” first, especially if you are a BoostAero XML beginner. The document is available for download on the SupplyOn AirSupply M2M Guidelines Download Area (http://www.supplyon.com/supplyon_download_area_m2m_guidelines.html).

# Purpose of the document

This document functions as a technical Guide for suppliers to connect to the new shared European aerospace Supply Chain Management Platform via a Machine-to-Machine connection. The technical Guide includes the XML structure definition for high level (business) context in order to provide suppliers with all necessary information.

M2M/EDI with the SCM Platform means EDI from/to a supplier to/from the SCM application; there is no direct EDI connection or direct message flow between a supplier and its customer without the SCM application.

Supplier integration of SupplyOn AirSupply data is possible via two formats: CSV and BoostAero XML V2.10 as a message format. In this document the XML format is documented.

Alongside with this technical guide you will receive the message format guidelines for the message type “VMI SELF BILLING INVOICE”.

## Self Billing Invoice Definition

A VMI Self Billing Invoice message is a billing document prepared and sent by the customer. The customer sends in correction documents to correct quantities billed in the invoice or retro-billing documents to adjust prices in the invoice.

## Preliminary Remarks

The following rules apply to all messages (if no exception is specified within the respective field description):

- Negative numbers: The minus sign must be in front of the digits (e.g. -10).

- Decimal numbers:

  - The delimiter is a point

  - Before the decimal point max. 15 digits are allowed

  - After the decimal point max. 5 digits are allowed

- Date format is CCYY-MM-DD.

- DATETIME format is CCYY-MM-DDThh:mm:ss.

- BOOLEAN values must be “true” or “false”

- INTEGER values must be whole numbers

- The currency will be described with 3 characters

## Composition of the specification

The data table describes the underelements of the affected element, which get listed sequently through the following columns:

- The first column contains the field name

- The second column describes the field type

- The column “length” describes the maximum character length of the element

- The column “level” describes the layer respectively position of the element within the message hierarchy levels. Please note, that the message hierarchy must not reflect the hierarchy itself as we use the standard.

- The column “example” contains possible sample dates to the affected element

- The column “status” specifies whether the element must (M) get applied or whether the usage is optional (C)

- The row “description” defines the assignment of the element

## Composition of the XML message graphics

The element, which gets described in the respective chapter, is marked blue and bold. Whether the element contains continuative elements, it gets assigned with a plus sign.

The elements have relations to each other, which has following values:

1	Stand for mandatory elements or fields within the affected element, which could only occur once

0..1	Stand for conditional elements or fields within the affected element, which are not required and must not get repeated

0..*	Stand for conditional elements or fields within the affected element, which are not required and could get repeated

1..*	Stand for mandatory elements or fields within the affected element, which could occur several times

## BoostAero Attribut Codes Lists

Within the document “BAI_Codes_Lists_V2 10_20110310.xls”, the code lists for the

corresponding elements including possible values are attached.

## Glossary

ARP	Airbus Resource Planning integrates internal and external management information across an entire organization

ASRAM	stands for Aero Spatial (context) Reusable Assembly Messages. The UN/CEFACT NDR (XML Name and Design Rules) impose us to use this namespace for all components. The namespace must have a minimum of 3 characters (we use 5) and these characters must represent a clear acronym.

as	Aero Spatial

C		Conditional field, which don´t requires the entry of any data

CIBI		Cross Industry Billing Invoice

CII		Cross Industry Invoice

EDI	Electronic Data Interchange: the computer-to-computer exchange of structured information, by agreed message standards, from one computer application to another by electronic means

M		Mandatory field is required and must get completed with data

M2M	Machine-to-Machine is a term used to describe marketplace-to-marketplace connectivity which enables the creation of global e-business networks.

SCM	Supply Chain Management is the process of planning, implementing, and controlling the operations of the supply chain as efficiently as possible

CI_Self-BillingInvoice

The header is specified by the item CI_Self-BillingInvoice consists of following substructures:

- as:CIExchangedDocumentContext

- as:CIBISupplyChainTradeTransaction

- as:CIIExchangedDocument

1

as:CI_Self-BillingInvoice

1

1

as:CIBISupplyChainTradeTransaction

as:CIExchangedDocumentContext

as:CIIExchangedDocument

Explanation

“as:CIExchangedDocumentContext” contains message exchange specific context data like message-ID or creation timestamp. This data is added by SupplyOn to the message. “as:CIBISupplyChainTradeTransaction” contains business address data of the contracting parties (customer, supplier,..) as well as data and references of the agreement contract. “as:CIIExchangedDocument” contains data for the transmitted document like „document number“, “last modification date“ , „sender and receiver information“ and others.

# as:CIExchangedDocumentContext

1

1

as:CIExchangedDocumentContext

asram:ApplicationSpecifiedCIDocumentContextParameter

asram:SpecifiedTransactionIdentifier

Explanation

“as:CIExchangedDocumentContext” contains specific context data like message-ID or creation timestamp. Origin of data is SupplyOn EAI infrastructure. (Seeburger BIS6).

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:SpecifiedTransactionIdentifier | String | 50 | 3 | 77777 | M |
| InternalMessageID: Internal SupplyOn message ID |  |  |  |  |  |

## asram:ApplicationSpecifiedCIDocumentContextParameter

0..11

asram:ApplicationSpecifiedCIDocumentContextParameter

1

asram:SpecifiedCIDocumentVersion

asram:TestIndicator

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:TestIndicator | BOOLEAN | - | 4 | true | C |
| TestFlag: Test flag shows whether message is a test-message possible values: true or false Not supported in Release 1. SupplyOn uses for test data a test system with different http partners for test and production. |  |  |  |  |  |

### asram:SpecifiedCIDocumentVersion

1

asram:SpecifiedCIDocumentVersion

asram:IssueDateTime

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IssueDateTime | DATETIME | - | 5 | 2011-05-31T13:20:05 | M |
| CreationTimeStamp: The customer´s ERP creation date and time of the demand forecast. |  |  |  |  |  |

# as:CIBISupplyChainTradeTransaction

The header is specified by the item as:CIBISupplyChainTradeTransaction consists of following substructures:

3.1  asram:ApplicableCIBIHSupplyChainTradeAgreement

3.2  asram:ApplicableCIBIHSupplyChainTradeSettlement

3.3  asram:IncludedCIBILSupplyChainTradeLineItem

0..11

as:CIBISupplyChainTradeTransaction

1

1

asram:ApplicableCIBIHSupplyChainTradeSettlement

asram:ApplicableCIBIHSupplyChainTradeAgreement

asram:IncludedCIBILSupplyChainTradeLineItem

Explanation

“asram:ApplicableCIBIHSupplyChainTradeAgreement” contains business address data of the contracting parties customer and supplier. “asram:ApplicableCIBIHSupplyChainTradeSettlement” contains business contract, price and payment data. “asram:IncludedCIBILSupplyChainTradeLineItem” contains specific information about the ordered material.

## asram:ApplicableCIBIHSupplyChainTradeAgreement

0..11

0..11

asram:ApplicableCIBIHSupplyChainTradeAgreement

asram:SellerCITradeParty

asram:BuyerCITradeParty

Explanation

“asram:ApplicableCIBIHSupplyChainTradeAgreement” contains business address data of the contracting parties customer and supplier.

### asram:BuyerCITradeParty

0..11

asram:GlobalIdentificationIdentifier

asram:BuyerCITradeParty

0..11

0..*

0..11

0..11

asram:DefinedCITradeContact

asram:DUNSIdentificationIdentifier

asram:SpecifiedCITaxRegistration

asram:PostalCITradeAddress

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DUNSIdentificationIdentifier | String | 35 | 5 | 351200012 | C |
| CustomerDunsNumber: The Data Universal Numbering System, abbreviated as DUNS or D-U-N-S is a system developed and regulated by Dun & Bradstreet (D&B) which assigns a unique numeric identifier to a single business entity. This numeric identifier is then referred to as a DUNS number. |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 5 | 48211 | C |
| CustomerPartnerNumber |  |  |  |  |  |

#### asram:SpecifiedCITaxRegistration

##### VATIdentNo

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | ID123 | C |
| VATIdentNo(Customer): The European Union requires a VAT (value added tax) identification number on invoices between entities registered for VAT |  |  |  |  |  |

##### TaxNumber

attributes

schemeName

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | 220115 | C |
| If attribute “schemeName” is equal to value “"Local”, the following assignment is valid: TaxNumber(Customer): A local identification number assigned to a business by a state's revenue department |  |  |  |  |  |

#### asram:PostalCITradeAddress

0..11

0..11

0..11

asram:PostalCITradeAddress

0..11

0..1

0..11

0..11

asram:PostOfficeBoxText

asram:PostcodeCode

asram:StreetNameText

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

0..11

0..11

0..11

0..11

0..11

asram:CountryIdentifier

asram:CityNameText

asram:CountryNameText

asram:CountrySub-DivisionNameText

asram:LineFiveText

asram:LineFourText

asram:LineOneText

asram:LineThreeText

asram:LineTwoText

Explanation

“asram:PostalCITradeAddress” contains business address data of the corresponding customer.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CityNameText | String | 40 | 6 | City1 | C |
| City: Contains of the contact address: City. |  |  |  |  |  |
| asram:CountryIdentifier | String | 3 | 6 | ABC | C |
| Countrycode: Contains of the contact address: Country (code). |  |  |  |  |  |
| asram:CountryNameText | String | 35 | 6 | Country1 | C |
| Country |  |  |  |  |  |
| asram:CountrySub-DivisionNameText | String | 40 | 6 | CountryS | C |
| State: Contains of the contact address: State. |  |  |  |  |  |
| asram:LineFiveText | String | 10 | 6 | 12340 | C |
| Postal code Postbox: Contains of the contact address: postbox ZIP code |  |  |  |  |  |
| asram:LineFourText | String | 64 | 6 | Adress4 | C |
| Name4: Contains of the address: Name 4. |  |  |  |  |  |
| asram:LineOneText | String | 64 | 6 | Adress | C |
| Name1: Contains of the address: Name. |  |  |  |  |  |
| asram:LineThreeText | String | 64 | 6 | Adress3 | C |
| Name3: Contains of the address: Name 3. |  |  |  |  |  |
| asram:LineTwoText | String | 64 | 6 | Adress2 | C |
| Name2: Contains of the address: Name 2. |  |  |  |  |  |
| asram:PostOfficeBoxText | String | 10 | 6 | B123 | C |
| Postbox: Contains of the postbox |  |  |  |  |  |
| asram:PostcodeCode | String | 10 | 6 | 14523 | C |
| Postalcode: Contains of the contact address: postal code. |  |  |  |  |  |
| asram:StreetNameText | String | 40 | 6 | Street | C |
| Street: The first occurrence contains Street: Contains of the address: Street. Street2: The first occurrence contains Street2: Contains the continuing of the address: Street. |  |  |  |  |  |

##### asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

asram:TimeZoneCodeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:TimeZoneCodeCode | String | 50 | 7 | Zone1 | C |
| CustomerTime Zone: Contains of the customer contact local timezone |  |  |  |  |  |

#### asram:DefinedCITradeContact

0..*

asram:TelephoneCIUniversalCommunication

0..11

asram:TitleText

0..*

0..11

asram:DefinedCITradeContact

0..*

asram:EmailURICIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:DepartmentNameText

asram:PersonNameText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DepartmentNameText | String | 35 | 6 | DepName | C |
| CustomerContactDepartment: Contains of the customer contact department |  |  |  |  |  |
| asram:PersonNameText | String | 35 | 6 | PersName | C |
| CustomerContactName: Contains of the customer contact name |  |  |  |  |  |
| asram:TitleText | String | 15 | 6 | Dr. | C |
| CustomerContactTitle: Contains of the customer contact title |  |  |  |  |  |

##### asram:EmailURICIUniversalCommunication

asram:EmailURICIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 64 | 7 | abc@abc.fr | C |
| CustomerContactMail: Contains of the contact mail |  |  |  |  |  |

##### asram:FaxCIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| CustomerContactFax: Contains of the contact fax |  |  |  |  |  |

##### asram:TelephoneCIUniversalCommunication

asram:TelephoneCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| CustomerContactPhone: Contains of the contact phone |  |  |  |  |  |

### asram:SellerCITradeParty

0..11

asram:GlobalIdentificationIdentifier

asram:SellerCITradeParty

0..11

0..*

0..11

0..11

asram:DefinedCITradeContact

asram:DUNSIdentificationIdentifier

asram:SpecifiedCITaxRegistration

asram:PostalCITradeAddress

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DUNSIdentificationIdentifier | String | 35 | 5 | 351200012 | C |
| SupplierDunsNumber: The Data Universal Numbering System, abbreviated as DUNS or D-U-N-S is a system developed and regulated by Dun & Bradstreet (D&B) which assigns a unique numeric identifier to a single business entity. This numeric identifier is then referred to as a DUNS number. |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 5 | 48211 | C |
| SupplierPartnerNumber |  |  |  |  |  |

#### asram:SpecifiedCITaxRegistration

##### VATIdentNo

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | ID123 | C |
| VATIdentNo(Supplier): The European Union requires a VAT (value added tax) identification number on invoices between entities registered for VAT |  |  |  |  |  |

##### TaxNumber

attributes

schemeName

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | 220115 | C |
| If attribute “schemeName” is equal to value “"Local”, the following assignment is valid: TaxNumber(Supplier): A local identification number assigned to a business by a state's revenue department |  |  |  |  |  |

#### asram:PostalCITradeAddress

0..11

0..11

0..11

asram:PostalCITradeAddress

0..11

0..1

0..11

0..11

asram:PostOfficeBoxText

asram:PostcodeCode

asram:StreetNameText

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

0..11

0..11

0..11

0..11

0..11

asram:CountryIdentifier

asram:CityNameText

asram:CountryNameText

asram:CountrySub-DivisionNameText

asram:LineFiveText

asram:LineFourText

asram:LineOneText

asram:LineThreeText

asram:LineTwoText

Explanation

“asram:PostalCITradeAddress” contains business address data of the corresponding supplier.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CityNameText | String | 40 | 6 | City1 | C |
| City: Contains of the contact address: City. |  |  |  |  |  |
| asram:CountryIdentifier | String | 3 | 6 | ABC | C |
| Countrycode: Contains of the contact address: Country (code). |  |  |  |  |  |
| asram:CountryNameText | String | 35 | 6 | Country1 | C |
| Country |  |  |  |  |  |
| asram:CountrySub-DivisionNameText | String | 40 | 6 | CountryS | C |
| State: Contains of the contact address: State. |  |  |  |  |  |
| asram:LineFiveText | String | 10 | 6 | 12340 | C |
| Postal code Postbox: Contains of the contact address: postbox ZIP code |  |  |  |  |  |
| asram:LineFourText | String | 64 | 6 | Adress4 | C |
| Name4: Contains of the address: Name 4. |  |  |  |  |  |
| asram:LineOneText | String | 64 | 6 | Adress | C |
| Name1: Contains of the address: Name. |  |  |  |  |  |
| asram:LineThreeText | String | 64 | 6 | Adress3 | C |
| Name3: Contains of the address: Name 3. |  |  |  |  |  |
| asram:LineTwoText | String | 64 | 6 | Adress2 | C |
| Name2: Contains of the address: Name 2. |  |  |  |  |  |
| asram:PostOfficeBoxText | String | 10 | 6 | B123 | C |
| Postbox: Contains of the postbox |  |  |  |  |  |
| asram:PostcodeCode | String | 10 | 6 | 14523 | C |
| Postalcode: Contains of the contact address: postal code. |  |  |  |  |  |
| asram:StreetNameText | String | 40 | 6 | Street | C |
| Street: The first occurrence contains Street: Contains of the address: Street. Street2: The first occurrence contains Street2: Contains the continuing of the address: Street. |  |  |  |  |  |

##### asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

asram:TimeZoneCodeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:TimeZoneCodeCode | String | 50 | 7 | Zone1 | C |
| SupplierTime Zone: Contains of the supplier contact local timezone |  |  |  |  |  |

#### asram:DefinedCITradeContact

0..*

asram:TelephoneCIUniversalCommunication

0..11

asram:TitleText

0..*

0..11

asram:DefinedCITradeContact

0..*

asram:EmailURICIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:DepartmentNameText

asram:PersonNameText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DepartmentNameText | String | 35 | 6 | DepName | C |
| SupplierContactDepartment: Contains of the supplier contact department |  |  |  |  |  |
| asram:PersonNameText | String | 35 | 6 | PersName | C |
| SupplierContactName: Contains of the supplier contact name |  |  |  |  |  |
| asram:TitleText | String | 15 | 6 | Dr. | C |
| SupplierContactTitle: Contains of the supplier contact title |  |  |  |  |  |

##### asram:EmailURICIUniversalCommunication

asram:EmailURICIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 64 | 7 | abc@abc.fr | C |
| SupplierContactMail: Contains of the contact mail |  |  |  |  |  |

##### asram:FaxCIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| SupplierContactFax: Contains of the contact fax |  |  |  |  |  |

##### asram:TelephoneCIUniversalCommunication

asram:TelephoneCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| SupplierContactPhone: Contains of the contact phone |  |  |  |  |  |

## asram:ApplicableCIBIHSupplyChainTradeSettlement

The header is specified by the item asram:ApplicableCIBIHSupplyChainTradeSettlement consists of following substructures:

- asram:DescriptionText

- asram:PaymentAmount

- asram:PaymentCurrencyCode

- asram:TaxCurrencyCode

- asram:TotalInvoiceAmount

3.2.1  asram:AcceptanceReferencedCIReferencedDocument

3.2.2  asram:InvoiceeCITradeParty

3.2.3  asram:PayeeCITradeParty

3.2.4  asram:SpecifiedCITradePaymentTerms

3.2.5  asram:SubtotalCalculatedCITradeTax

attributes

currencyID

attributes

currencyID

asram:ApplicableCIBIHSupplyChainTradeSettlement

0..11

0..11

0..11

asram:SubtotalCalculatedCITradeTax

0..11

0..11

0..11

0..11

0..11

0..11

0..11

asram:PaymentAmount

asram:DescriptionText

asram:PaymentCurrencyCode

asram:TaxCurrencyCode

asram:TotalInvoiceAmount

asram:AcceptanceReferencedCIReferencedDocument

asram:InvoiceeCITradeParty

asram:PayeeCITradeParty

asram:SpecifiedCITradePaymentTerms

Explanation

“asram:ApplicableCIBIHSupplyChainTradeSettlement” contains business contract, price and payment data.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DescriptionText | String | 70 | 4 | doc légal - a voir | C |
| StatementOfLiability: Factor Statement of Liability |  |  |  |  |  |
| asram:PaymentAmount | Decimal | 15+5 | 4 | 12 | C |
| GrossTotalAmount |  |  |  |  |  |
| currencyID | String | 3 | 4 | EUR | C |
| Currency |  |  |  |  |  |
| asram:PaymentCurrencyCode | String | 3 | 4 | EUR | C |
| Currency |  |  |  |  |  |
| asram:TaxCurrencyCode | String | 3 | 4 | EUR | C |
| VATLocalAmountCurrency: VAT Local amount Currency (Euro or Sterling) |  |  |  |  |  |
| asram:TotalInvoiceAmount | Decimal | 15+5 | 4 | 12 | C |
| NetTotalAmount |  |  |  |  |  |

### asram:AcceptanceReferencedCIReferencedDocument

1

asram:AcceptanceReferencedCIReferencedDocument

asram:GlobalIdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 5 | 4904817 | C |
| AccountDocumentNumber |  |  |  |  |  |

### asram:InvoiceeCITradeParty

asram:InvoiceeCITradeParty

0..11

asram:GlobalIdentificationIdentifier

0..11

0..*

0..11

0..11

asram:DefinedCITradeContact

asram:DUNSIdentificationIdentifier

asram:SpecifiedCITaxRegistration

asram:PostalCITradeAddress

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DUNSIdentificationIdentifier | String | 35 | 5 | 351200012 | C |
| InvoiceeDunsNumber: The Data Universal Numbering System, abbreviated as DUNS or D-U-N-S is a system developed and regulated by Dun & Bradstreet (D&B) which assigns a unique numeric identifier to a single business entity. This numeric identifier is then referred to as a DUNS number |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 5 | 48211 | C |
| InvoiceePartnerNumber |  |  |  |  |  |

#### asram:SpecifiedCITaxRegistration

##### VATIdentNo

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | ID123 | C |
| VATIdentNo(Invoicee): The European Union requires a VAT (value added tax) identification number on invoices between entities registered for VAT |  |  |  |  |  |

##### TaxNumber

attributes

schemeName

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | 220115 | C |
| If attribute “schemeName” is equal to value “"Local”, the following assignment is valid: TaxNumber(Invoicee): A local identification number assigned to a business by a state's revenue department |  |  |  |  |  |

#### asram:PostalCITradeAddress

0..11

0..11

0..11

asram:PostalCITradeAddress

0..11

0..1

0..11

0..11

asram:PostOfficeBoxText

asram:PostcodeCode

asram:StreetNameText

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

0..11

0..11

0..11

0..11

0..11

asram:CountryIdentifier

asram:CityNameText

asram:CountryNameText

asram:CountrySub-DivisionNameText

asram:LineFiveText

asram:LineFourText

asram:LineOneText

asram:LineThreeText

asram:LineTwoText

Explanation

“asram:PostalCITradeAddress” contains business address data of the corresponding invoicee.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CityNameText | String | 40 | 6 | City1 | C |
| City: Contains of the contact address: City. |  |  |  |  |  |
| asram:CountryIdentifier | String | 3 | 6 | ABC | C |
| Countrycode: Contains of the contact address: Country (code). |  |  |  |  |  |
| asram:CountryNameText | String | 35 | 6 | Country1 | C |
| Country |  |  |  |  |  |
| asram:CountrySub-DivisionNameText | String | 40 | 6 | CountryS | C |
| State: Contains of the contact address: State. |  |  |  |  |  |
| asram:LineFiveText | String | 10 | 6 | 12340 | C |
| Postal code Postbox: Contains of the contact address: postbox ZIP code |  |  |  |  |  |
| asram:LineFourText | String | 64 | 6 | Adress4 | C |
| Name4: Contains of the address: Name 4. |  |  |  |  |  |
| asram:LineOneText | String | 64 | 6 | Adress | C |
| Name1: Contains of the address: Name. |  |  |  |  |  |
| asram:LineThreeText | String | 64 | 6 | Adress3 | C |
| Name3: Contains of the address: Name 3. |  |  |  |  |  |
| asram:LineTwoText | String | 64 | 6 | Adress2 | C |
| Name2: Contains of the address: Name 2. |  |  |  |  |  |
| asram:PostOfficeBoxText | String | 10 | 6 | B123 | C |
| Postbox: Contains of the postbox |  |  |  |  |  |
| asram:PostcodeCode | String | 10 | 6 | 14523 | C |
| Postalcode: Contains of the contact address: postal code. |  |  |  |  |  |
| asram:StreetNameText | String | 40 | 6 | Street | C |
| Street: The first occurrence contains Street: Contains of the address: Street. Street2: The first occurrence contains Street2: Contains the continuing of the address: Street. |  |  |  |  |  |

##### asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

asram:TimeZoneCodeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:TimeZoneCodeCode | String | 50 | 7 | Zone1 | C |
| InvoiceeTime Zone: Contains of the invoicee contact local timezone |  |  |  |  |  |

#### asram:DefinedCITradeContact

0..*

asram:TelephoneCIUniversalCommunication

0..11

asram:TitleText

0..*

0..11

asram:DefinedCITradeContact

0..*

asram:EmailURICIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:DepartmentNameText

asram:PersonNameText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DepartmentNameText | String | 35 | 6 | DepName | C |
| InvoiceeContactDepartment: Contains of the invoicee contact department |  |  |  |  |  |
| asram:PersonNameText | String | 35 | 6 | PersName | C |
| InvoiceeContactName: Contains of the invoicee contact name |  |  |  |  |  |
| asram:TitleText | String | 15 | 6 | Dr. | C |
| InvoiceeContactTitle: Contains of the invoicee title |  |  |  |  |  |

##### asram:EmailURICIUniversalCommunication

asram:EmailURICIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 64 | 7 | abc@abc.fr | C |
| InvoiceeContactMail: Contains of the contact email |  |  |  |  |  |

##### asram:FaxCIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| InvoiceeContactFax: Contains of the contact fax |  |  |  |  |  |

##### asram:TelephoneCIUniversalCommunication

asram:TelephoneCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146201 | C |
| InvoiceeContactPhone: Contains of the contact phone |  |  |  |  |  |

### asram:PayeeCITradeParty

asram:PayeeCITradeParty

0..11

asram:GlobalIdentificationIdentifier

0..11

0..*

0..11

0..11

asram:DefinedCITradeContact

asram:DUNSIdentificationIdentifier

asram:SpecifiedCITaxRegistration

asram:PostalCITradeAddress

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DUNSIdentificationIdentifier | String | 35 | 5 | 351200012 | C |
| PayeeDunsNumber: The Data Universal Numbering System, abbreviated as DUNS or D-U-N-S is a system developed and regulated by Dun & Bradstreet (D&B) which assigns a unique numeric identifier to a single business entity. This numeric identifier is then referred to as a DUNS number |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 5 | 48211 | C |
| PayeePartnerNumber |  |  |  |  |  |

#### asram:SpecifiedCITaxRegistration

##### VATIdentNo

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | ID123 | C |
| VATIdentNo(Payee): The European Union requires a VAT (value added tax) identification number on invoices between entities registered for VAT |  |  |  |  |  |

##### TaxNumber

attributes

schemeName

asram:SpecifiedCITaxRegistration

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 6 | 220115 | C |
| If attribute “schemeName” is equal to value “"Local”, the following assignment is valid: TaxNumber(Payee): A local identification number assigned to a business by a state's revenue department |  |  |  |  |  |

#### asram:PostalCITradeAddress

0..11

0..11

0..11

asram:PostalCITradeAddress

0..11

0..1

0..11

0..11

asram:PostOfficeBoxText

asram:PostcodeCode

asram:StreetNameText

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

0..11

0..11

0..11

0..11

0..11

asram:CountryIdentifier

asram:CityNameText

asram:CountryNameText

asram:CountrySub-DivisionNameText

asram:LineFiveText

asram:LineFourText

asram:LineOneText

asram:LineThreeText

asram:LineTwoText

Explanation

“asram:PostalCITradeAddress” contains business address data of the corresponding payee.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CityNameText | String | 40 | 6 | City1 | C |
| City: Contains of the contact address: City. |  |  |  |  |  |
| asram:CountryIdentifier | String | 3 | 6 | ABC | C |
| Countrycode: Contains of the contact address: Country (code). |  |  |  |  |  |
| asram:CountryNameText | String | 35 | 6 | Country1 | C |
| Country |  |  |  |  |  |
| asram:CountrySub-DivisionNameText | String | 40 | 6 | CountryS | C |
| State: Contains of the contact address: State. |  |  |  |  |  |
| asram:LineFiveText | String | 10 | 6 | 12340 | C |
| Postal code Postbox: Contains of the contact address: postbox ZIP code |  |  |  |  |  |
| asram:LineFourText | String | 64 | 6 | Adress4 | C |
| Name4: Contains of the address: Name 4. |  |  |  |  |  |
| asram:LineOneText | String | 64 | 6 | Adress | C |
| Name1: Contains of the address: Name. |  |  |  |  |  |
| asram:LineThreeText | String | 64 | 6 | Adress3 | C |
| Name3: Contains of the address: Name 3. |  |  |  |  |  |
| asram:LineTwoText | String | 64 | 6 | Adress2 | C |
| Name2: Contains of the address: Name 2. |  |  |  |  |  |
| asram:PostOfficeBoxText | String | 10 | 6 | B123 | C |
| Postbox: Contains of the postbox |  |  |  |  |  |
| asram:PostcodeCode | String | 10 | 6 | 14523 | C |
| Postalcode: Contains of the contact address: postal code. |  |  |  |  |  |
| asram:StreetNameText | String | 40 | 6 | Street | C |
| Street: The first occurrence contains Street: Contains of the address: Street. Street2: The first occurrence contains Street2: Contains the continuing of the address: Street. |  |  |  |  |  |

##### asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

asram:Geo-CoordinateIdentificationCIGeographicalCoordinate

0..11

asram:TimeZoneCodeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:TimeZoneCodeCode | String | 50 | 7 | Zone1 | C |
| PayeeTime Zone: Contains of the payee contact local timezone |  |  |  |  |  |

#### asram:DefinedCITradeContact

0..*

asram:TelephoneCIUniversalCommunication

0..11

asram:TitleText

0..*

0..11

asram:DefinedCITradeContact

0..*

asram:EmailURICIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:DepartmentNameText

asram:PersonNameText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DepartmentNameText | String | 35 | 6 | DepName | C |
| PayeeContactDepartment: Contains of the payee contact department |  |  |  |  |  |
| asram:PersonNameText | String | 35 | 6 | PersName | C |
| PayeeContactName: Contains of the payee contact name |  |  |  |  |  |
| asram:TitleText | String | 15 | 6 | Dr. | C |
| PayeeContactTitle: Contains of the payee title |  |  |  |  |  |

##### asram:EmailURICIUniversalCommunication

asram:EmailURICIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 64 | 7 | abc@abc.fr | C |
| PayeeContactMail: Contains of the contact email |  |  |  |  |  |

##### asram:FaxCIUniversalCommunication

asram:FaxCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146200 | C |
| PayeeContactFax: Contains of the contact fax |  |  |  |  |  |

##### asram:TelephoneCIUniversalCommunication

asram:TelephoneCIUniversalCommunication

0..11

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CompleteNumberText | String | 25 | 7 | +33 146201 | C |
| PayeeContactPhone: Contains of the contact phone |  |  |  |  |  |

### asram:SpecifiedCITradePaymentTerms

asram:SpecifiedCITradePaymentTerms

0..11

0..11

0..11

asram:DueDateDateTime

asram:DescriptionText

asram:InstructionTypeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:DescriptionText | String | 70 | 5 | Payment immediately | C |
| TermsOfPaymentDescription |  |  |  |  |  |
| asram:DueDateDateTime | Date | 8 | 5 | 2011-04-01 | C |
| DueDate |  |  |  |  |  |
| asram:InstructionTypeCode | String | 17 | 5 | payment terms | C |
| TermsOfPayment |  |  |  |  |  |

### asram:SubtotalCalculatedCITradeTax

attributes

currencyID

1

1

asram:SubtotalCalculatedCITradeTax

asram:TypeCode

asram:CalculatedAmount

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CalculatedAmount | Decimal | 15+5 | 5 | 1.55 | C |
| If “asram:TypeCode” is equal to Value “TOT”, the following assignment is valid: VATTotalAmount |  |  |  |  |  |
| asram:CalculatedAmount | Decimal | 15+5 | 5 | 1.67 | C |
| If “asram:TypeCode” is equal to Value “LOC”, the following assignment is valid: VATLocalAmount |  |  |  |  |  |
| currencyID | String | 12 | 6 | EUR | C |
| Currency |  |  |  |  |  |

## asram:IncludedCIBILSupplyChainTradeLineItem

The header is specified by the item asram:IncludedCIBILSupplyChainTradeLineItem consists of following substructures:

- asram:IdentificationIdentifier

3.3.1  asram:SpecifiedCITradeProduct

3.3.2  asram:SpecifiedCIBILSupplyChainTradeAgreement

3.3.3  asram:SpecifiedCIBILSupplyChainTradeDelivery

3.3.4  asram:SpecifiedCIBILSupplyChainTradeSettlement

1

asram:IncludedCIBILSupplyChainTradeLineItem

0..11

0..11

0..11

0..11

asram:SpecifiedCIBILSupplyChainTradeSettlement

asram:SpecifiedCITradeProduct

asram:IdentificationIdentifier

asram:SpecifiedCIBILSupplyChainTradeAgreement

asram:SpecifiedCIBILSupplyChainTradeDelivery

Explanation

“asram:IncludedCIBILSupplyChainTradeLineItem” contains specific information about the ordered material.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 6 | 4 | 10 | C |
| ItemLineNumber |  |  |  |  |  |

### asram:SpecifiedCITradeProduct

1

1

asram:SpecifiedCITradeProduct

0..11

0..11

0..11

asram:IndividualCITradeProductInstance

asram:IdentificationIdentifier

asram:BuyerAssignedIdentificationIdentifier

asram:SellerAssignedIdentificationIdentifier

asram:InformationCINote

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:BuyerAssignedIdentificationIdentifier | String | 64 | 5 | 122016 | M |
| CustomerMaterialNumber |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 18 | 5 | 20100107 | M |
| SAPMaterialReference |  |  |  |  |  |
| asram:SellerAssignedIdentificationIdentifier | String | 64 | 5 | ASNA3311-7550 | C |
| SupplierMaterialNumber: Retrieved during import from article master data (if not found, value is taken from the PO) |  |  |  |  |  |

#### asram:InformationCINote

1

1

asram:InformationCINote

asram:SubjectCode

asram:ContentText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:ContentText | String | 256 | 6 | ECLISSE AVT.FERR.H/TE | C |
| If “asram:SubjectCode” is equal to Value “CMD”, the following assignment is valid: CustomerMaterialDescription |  |  |  |  |  |
| asram:ContentText | String | 256 | 6 | ECLISSE AVT.FERR.H/TE | C |
| If “asram:SubjectCode” is equal to Value “SMD”, the following assignment is valid: SupplierMaterialDescription: Supplier Material description. Retrieved during import from article master data (if not found, value is taken from the PO) |  |  |  |  |  |
| asram:ContentText | String | 10 | 6 | 08Y3041 | C |
| If “asram:SubjectCode” is equal to Value “CBN”, the following assignment is valid: BatchNumberCustomer |  |  |  |  |  |
| asram:ContentText | String | 15 | 6 | no data | C |
| If “asram:SubjectCode” is equal to Value “SBN”, the following assignment is valid: BatchNumberSupplier |  |  |  |  |  |

#### asram:IndividualCITradeProductInstance

asram:IndividualCITradeProductInstance

0..11

0..11

asram:SupplierAssignedSerialIdentificationIdentifier

asram:ExpiryDateTime

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:ExpiryDateTime | Date | 8 | 6 | 2011-04-01 | C |
| ExpiryDate: Date at which the product can no more be used. |  |  |  |  |  |
| asram:SupplierAssignedSerialIdentificationIdentifier | String | 18 | 6 | 9905 | C |
| SerialNumber |  |  |  |  |  |

### asram:SpecifiedCIBILSupplyChainTradeAgreement

1

0..11

asram:SpecifiedCIBILSupplyChainTradeAgreement

1

0..11

0..11

asram:AgreedPriceProductCITradePrice

asram:AdditionalReferencedCIReferencedDocument

asram:ProgramCIMissionProject

asram:BuyerCITradeParty

asram:SellerCITradeParty

#### asram:ProgramCIMissionProject

asram:ProgramCIMissionProject

0..11

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 16 | 6 | LR/0000009152 | C |
| Program/MSN: Defines the Aircraft type and Material Serial Number. |  |  |  |  |  |

#### asram:AdditionalReferencedCIReferencedDocument

1

1

1

asram:AdditionalReferencedCIReferencedDocument

asram:LineIdentifier

asram:GlobalIdentificationIdentifier

asram:TypeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:GlobalIdentificationIdentifier | String | 35 | 6 | 1200131525 | C |
| If “asram:TypeCode” is equal to Value “78”, the following assignment is valid: OrderNumber / VMI Reference |  |  |  |  |  |
| asram:LineIdentifier | Integer | - | 6 | 110 | C |
| If “asram:TypeCode” is equal to Value “78”, the following assignment is valid: OrderItem / VMIItemReference |  |  |  |  |  |

#### asram:BuyerCITradeParty

1

asram:BuyerCITradeParty

0..11

asram:DefinedCITradeContact

asram:SpecifiedCILegalOrganization

##### asram:SpecifiedCILegalOrganization

asram:SpecifiedCILegalOrganization

1

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 5 | 7 | Airbus | M |
| CustomerGroupCode: Buyer Corporate Group Identifier for M2M (EDI with suppliers). |  |  |  |  |  |

##### asram:DefinedCITradeContact

1

1

1

asram:DefinedCITradeContact

0..*

asram:TelephoneCIUniversalCommunication

asram:PersonNameText

asram:IdentificationIdentifier

asram:TypeCode

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 20 | 7 | 842 | C |
| If “asram:TypeCode” is equal to Value “OC”, the following assignment is valid: OrderingOfficerCode |  |  |  |  |  |
| asram:PersonNameText | String | 64 | 7 | RAUD Marylin | C |
| If “asram:TypeCode” is equal to Value “OC”, the following assignment is valid: OrderingOfficerName |  |  |  |  |  |

###### asram:TelephoneCIUniversalCommunication

1

asram:TelephoneCIUniversalCommunication

asram:CompleteNumberText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 24 | 8 | +33.5.61.18.99.99 | C |
| If “asram:TypeCode” is equal to Value “OC”, the following assignment is valid: OrderingOfficerPhone |  |  |  |  |  |

#### asram:SellerCITradeParty

1

asram:SellerCITradeParty

0..11

asram:DefinedCITradeContact

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 64 | 6 | 0000172763 | M |
| SupplierNumber: Defines the Supplier Airbus Resource Planing (ARP) system id |  |  |  |  |  |

##### asram:DefinedCITradeContact

asram:DefinedCITradeContact

1

1

asram:TypeCode

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 64 | 7 | 0000172763 | C |
| If “asram:TypeCode” is equal to Value “SC”, the following assignment is valid: SupplierPlannerCode: Stored by the supplier in article master data |  |  |  |  |  |

#### asram:AgreedPriceProductCITradePrice

attributes

currencyID

attributes

unitCode

asram:AgreedPriceProductCITradePrice

0..11

0..11

asram:UnitAmount

asram:BasisQuantity

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| unitCode | String | 3 | 6 | PCE | C |
| UnitOfMeasure |  |  |  |  |  |
| asram:BasisQuantity | Decimal | 15+5 | 6 | EUR | C |
| UnitPrice |  |  |  |  |  |
| asram:UnitAmount | Decimal | 15+5 | 6 | EUR | C |
| PricePerUnit |  |  |  |  |  |
| currencyID | String | 3 | 6 | EUR | C |
| Currency |  |  |  |  |  |

### asram:SpecifiedCIBILSupplyChainTradeDelivery

The header is specified by the item asram:SpecifiedCIBILSupplyChainTradeDelivery consists of following substructures:

- asram:ReceivedQuantity

3.3.3.1  asram:InformationCINote

3.3.3.2  asram:DespatchAdviceReferencedCIReferencedDocument

3.3.3.3  asram:ConsumptionCISupplyChainSchedule

3.3.3.4  asram:ShipToCITradeParty

attributes

unitCode

1

0..11

0..11

0..11

0..11

asram:SpecifiedCIBILSupplyChainTradeDelivery

asram:InformationCINote

asram:ReceivedQuantity

asram:DespatchAdviceReferencedCIReferencedDocument

asram:ConsumptionCISupplyChainSchedule

asram:ShipToCITradeParty

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:ReceivedQuantity | Decimal | 15+5 | 5 | 12 | C |
| DeliveryQuantity |  |  |  |  |  |
| unitCode | String | 3 | 5 | PCE | M |
| UnitOfMeasure(UOM) |  |  |  |  |  |

#### asram:InformationCINote

1

1

asram:InformationCINote

asram:SubjectCode

asram:ContentText

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:ContentText | String | 35 | 6 | 563142 | C |
| If “asram:SubjectCode” is equal to Value “DN”, the following assignment is valid: DeliveryNote |  |  |  |  |  |
| asram:ContentText | String | 4 | 6 | SG05 | C |
| If “asram:SubjectCode” is equal to Value “SL”, the following assignment is valid: StorageLocation: Customer ERP storage location. |  |  |  |  |  |

#### asram:DespatchAdviceReferencedCIReferencedDocument

asram:DespatchAdviceReferencedCIReferencedDocument

0..11

asram:IssuerAssignedIdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IssuerAssignedIdentificationIdentifier | String | 35 | 6 | 8610 | C |
| DeliveryNumber |  |  |  |  |  |

#### asram:ConsumptionCISupplyChainSchedule

asram:ConsumptionCISupplyChainSchedule

0..11

0..11

asram:ActualCISupplyChainEvent

asram:OccurrenceDateTime

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:OccurrenceDateTime | Date | 8 | 6 | 2011-04-01 | C |
| GoodsMovementDate |  |  |  |  |  |

##### asram:ActualCISupplyChainEvent

attributes

unitCode

asram:ActualCISupplyChainEvent

0..11

asram:UnitQuantity

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:UnitQuantity | Decimal | 15+5 | 7 | 1234 | C |
| GoodsMovementQuantity |  |  |  |  |  |
| unitCode | String | 3 | 7 | PCE | M |
| UnitOfMeasure(UOM) |  |  |  |  |  |

#### asram:ShipToCITradeParty

1

asram:ShipToCITradeParty

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 20 | 6 | TOF | M |
| CustomerPlantCode: Defines the plant code of the customer. |  |  |  |  |  |

### asram:SpecifiedCIBILSupplyChainTradeSettlement

asram:SpecifiedCIBILSupplyChainTradeSettlement

0..11

0..11

0..11

asram:SpecifiedCITradeAllowanceCharge

asram:SpecifiedCITradeSettlementMonetarySummation

asram:ApplicableCITradeTax

#### asram:SpecifiedCITradeSettlementMonetarySummation

attributes

currencyID

attributes

currencyID

asram:SpecifiedCITradeSettlementMonetarySummation

0..11

0..11

asram:TaxBasisTotalAmount

asram:LineTotalAmount

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:LineTotalAmount | Decimal | 15+5 | 6 | 1493.10 | C |
| TotalNetAmount |  |  |  |  |  |
| asram:TaxBasisTotalAmount | Decimal | 15+5 | 6 | 1200.00 | C |
| NetAmount |  |  |  |  |  |

#### asram:SpecifiedCITradeAllowanceCharge

attributes

currencyID

asram:SpecifiedCITradeAllowanceCharge

0..11

asram:ActualAmount

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:ActualAmount | String | 35 | 6 | 5 | C |
| SurchargesAndRebates: Contains values, which increase or decrease the amount of paid money. |  |  |  |  |  |
| currencyID | String | 12 | 6 | EUR | C |
| Currency |  |  |  |  |  |

#### asram:ApplicableCITradeTax

attributes

currencyID

0..11

0..11

asram:CalculatedAmount

asram:ApplicableCITradeTax

0..11

0..11

asram:CategoryNameText

asram:CategoryCode

asram:RateApplicablePercent

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:CalculatedAmount | String | 18 | 6 | 19 | C |
| TaxValue |  |  |  |  |  |
| currencyID | String | 12 | 6 | EUR | C |
| Currency |  |  |  |  |  |
| asram:CategoryCode | String | 2 | 6 | V5 | C |
| VATCode: VAT registration Code |  |  |  |  |  |
| asram:CategoryNameText | String | 50 | 6 | TVA déductible 19,6% | C |
| VATDescription: VAT registration Description |  |  |  |  |  |
| asram:RateApplicablePercent | String | 17 | 6 | 19 | C |
| TaxRate |  |  |  |  |  |

# as:CIIExchangedDocument

1

1

asram:SenderCITradeParty

0..11

0..11

0..11

0..11

as:CIIExchangedDocument

asram:RecipientCITradeParty

asram:IssueDateTime

asram:IdentificationIdentifier

asram:SenderAssignedIdentificationIdentifier

asram:SubmissionDateTime

Explanation

“as:CIIExchangedDocument” contains data for the transmitted document like „document number“, “last modification date“ , „sender and receiver information“ and others.

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 35 | 3 | 562132 | C |
| InvoiceNumber: Specific number of the invoice |  |  |  |  |  |
| asram:IssueDateTime | Date | 8 | 3 | 2011-04-05 | C |
| InvoiceDate: Date of creation of the invoice |  |  |  |  |  |
| asram:SenderAssignedIdentificationIdentifier | String | 16 | 3 | 1234428 | C |
| DocumentNumber: Specific number of the document |  |  |  |  |  |
| asram:SubmissionDateTime | Datetime | 14 | 3 | 2011-04-01 T00:01:20 | C |
| SentDateTime: Sent datetime (SAP datetime - Idoc creation) |  |  |  |  |  |

## asram:RecipientCITradeParty

1

asram:RecipientCITradeParty

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 70 | 4 | SupplyOn | M |
| ReceiverInfo: party who receives this XML message. No key, just for better traceability. |  |  |  |  |  |

## asram:SenderCITradeParty

1

asram:SenderCITradeParty

asram:IdentificationIdentifier

| Field Name | Field Type | Length | Level | Example | Status |
| --- | --- | --- | --- | --- | --- |
| Description |  |  |  |  |  |
| asram:IdentificationIdentifier | String | 70 | 4 | SupplyOn | M |
| SenderInfo / SystemOrigin : Party who sends this XML message (The system within the message gets created). No key, just for better traceability. |  |  |  |  |  |